Charging the car at its home charging station

Starting a charge

When nothing is plugged into the charging station, the LED display light on the station will pulse blue - showing it’s healthy.

A red pulse or no light indicates something is wrong, please report this

- Get the charging cable from the boot of the car

- Plug the charging cable into the charging station

- Open the charging flap on the nose of the car by pressing the car’s ‘release’ button

Button is down and right of the steering wheel

- Plug the other end of the charging cable into the car

- The LED light on the charging station will pulse green while the car initialises the charge. It will then turn a solid green to show it is charging.

You can also check that you have successfully started a charge by looking at the base of the car’s windscreen. 1, 2 or 3 blue charging lights will be flashing:

- Light 1 will flash when car is charging at 0-33%

- Light 1 will be solid and Light 2 will flash when the car is charging at 34-66%

- Lights 1&2 will be solid and Light 3 will flash when the car is charging at 67-99%

- When the car reaches 100% it stops charging and all 3 lights go out.

Once you have started the charge, remember to lock the car using your Co-wheels smartcard.

The car will charge at approximately 30miles/20% per hour.

Ending a charge

- ‘Unlock’ the cable by pressing the car’s ‘release’ button

Button is down and right of the steering wheel

- Unplug the cable from the car

- Unplug the cable from the charging station

- Store the cable in the boot of the car